Methods S1 PCR PCR Cycling Parameters

PCR reactions were carried out in 25ul reaction mixtures including 2.5ul of 10x HotstarTaq PCR buffer, 0.5ul of 2 mM of deoxynucleotide triphosphates (dNTPs), 1.0ul of 10 uM ILL-1F-primer, 0.13ul of HotStarTaq (5 units/ul), 0.25ul of 100x bovine serum albumin (BSA), 14.62ul of molecular grade water, and 2ul of the DNA template. PCR cycling parameters consisted of an initial cycle of 5 min at 95˚C for denaturation of the DNA, followed by 29 cycles consisting of 30s at 95˚C, 30s at 51˚C, and 1 min at 72˚C of annealing and 10 min at 72˚C for an extension. A negative and positive control were used during amplification to ensure amplification of target DNA and exclude contamination of the products. For samples that failed to amplify, DNA concentration were increased to 5ul (10:90 PCR product). PCR products were visualized with gel electrophoresis before submitting to Vincent J. Coates (VJC) Genomic Sequencing Laboratory at the University of California-Berkeley. The VJC laboratory cleaned the PCR product with AMPURE (Beckman Coulter Inc., Brea, CA, USA), quantified fluorescently with the Qubit dsDNA HS kit (Life Technologies Inc., Gaithersburg, MD, USA), and pooled at equimolar concentration. Quality check was performed with the Agilent 2100 Bioanalyzer (Agilent Technologies, Santa Clara, CA, USA) according to established protocols (Glassman, Wang and Bruns 2017), and sequenced with Illumina MiSeq PE 2 x 300.

Methods S1 Bioinformatics steps

Paired-end demultiplexed sequence files, containing 4,535,423 raw sequences in 213 samples were processed using Qiime2, version 2019.10 (Caporaso et al., 2019). The primers and adapter sequences from the 5’ were removed by the sequencing facility. The 3’ primer and adapters, CTTGGTCATTTAGAGGAAGTAA, were removed using the qiime2-cut-a-dapt plugin (Martin, 2011). Quality control to filter out chimeric sequences and low-quality regions was done by parsing the sequences through DADA2 and truncating the 3’ end to 150 bp (Callahan et al., 2016). The taxonomic analysis was performed by first training a taxonomic classifier using the UNITE reference database with clustering at ≥ 99%  reference levels (Kõljalg et al. 2005; UNITE, 2017). The sequences were then assigned taxonomy using the fitted classifier, classify-sklearn plugin (Pedregosa et al., 2011), resulting in a clustered ASV table with 99% similarity, resulting in a table with 8,587 ASV’s and a mean sample frequency of 17,164 sequences per sample for total fungi.

Assigning taxonomic guild

Since the ITS1F-ITS2 allows for the amplification of all fungal family members, the ASV raw table was parsed through FUNGuild (Nguyen et al., 2016) to assign guilds. The selection of ectomycorrhizal taxa only was performed by going through the funguild output to verify the taxonomic assignment. Ectomycorrhizal Funguild assignment was accepted at all three confidence rankings of FunGuild, including “possible,” “probable,” and “highly probable.” However, the lower the confidence level, e.i. “possible,” the sample was most likely classified with multiple lifestyles. In such cases, we assigned a single taxonomy based on their literature description, where 3+ assignments as EcM or saprobic fungi resulted in them being classified as ectomycorrhizal or saprobic in our ASV table. We used R studio to create an EcM and saprobic ASV table by selecting all taxa classified as EcM on the full fungal table. A similar protocol was followed to create a saprobic only table.

Ectomycorrhizal bioinformatics (rarefaction 55)

The ASV table with FUNGuild output was used in R to subset all ASV’s belonging to the EcM guild.  This resulted in a total of 889 ASV’s. This table was additionally rarefied to remove the bottom 7% of the sequences to account for uneven sequencing depth, but because the burned sites were, on average, low in comparison to the burned units, the sequencing depth had to be maintained on the lower spectrum in order minimize discarding samples. The raw table was rarefied in R to retain 13,790 (1.28%) features in 197 (93.81%) samples at the sampling depth of 70 sequences/samples, resulting in an EcM table with 629 ASV’s. Although we would have liked to retain a higher number of sequences, this was difficult to achieve as we would have lost too many of the burned samples. A similar protocol was performed to attain the saprobic table; however, more taxa were classified as having a saprobic lifestyle. Thus, we were able to rarefy to 2,200 sequences/sample, resulting in a saprobic table with 2850 ASV’s and 448,800 sequences/sample in 204 (95.77%) of the samples.  Note out of 216 samples, 3 samples failed to amplify, thus ASV tables were created from the amplification and sequencing of 213 samples.